# Core DA CC Quarterly report – Reading Guide

| Version | 0.2 |
| --- | --- |
| Date | December 2022 |

Version history

| Version | Date | Summary of changes |
| --- | --- | --- |
| 0.3 | March 2023 (as of Q4 2022 report) | Addition of footnotes related to: - Art. 27(5)(c) from Introduction - Art. 7(3)(b)(i)-(ii) from Chapter 1. Allocation constraints - Art. 20(13)(e) from Chapter 2. Reductions of cross-zonal capacity Addition of note related to the file QX_20XX_Reductions.xlsx, worksheet Reductions_02, column Presolved. For more details, please see dedicated section in the reading guide |
| 0.2 | December 2022 (as of Q3 2022 report) | Additions related to the introduction of occurrences of RAM<20% Fmax as part of Chapter 2. Reductions of cross-zonal capacity. List of affected files, for more details please see dedicated sections in the reading guide: - QX_20XX_Reductions.xlsx, worksheet Reductions_01 - QX_20XX_Reductions-statistics.xlsx, worksheet Reductions-statistics_03 Note on consideration of ALEGrO-related constraints from Chapter 1. Allocation constraints Additions on DAVinCy TSOs (Austrian, German and Dutch TSOs) particularities and impact on statistics. For more details, please see dedicated sections in the reading guide: - Extension of footnote 3 - QX_20XX_ Reductions-statistics_04_2 - Volume of applied reductions - Estimated loss of economic surplus of applied reductions |
| 0.1 | Q2 2022 report | First version |

Table of Contents

Introduction	3

1. Allocation constraints	4

1.1	Reporting obligations from DA CCM	4

1.2	Reading guide for raw data	4

2. Reductions of cross-zonal capacity	6

2.1	Reporting obligations from DA CCM	6

2.2	Reading guide for raw data and statistics files	9

3. Flows resulting from net positions resulting from SDAC	20

3.1	Reporting obligations from DA CCM	20

3.2	Reading guide for raw data	20

# Introduction

According to the Core DA CCM and the Core DA CCM 1st amendment, the quarterly reporting obligations following the implementation of this methodology are as follows:

Art. 27(5): “The CCC, with the support of the Core TSOs where relevant, shall draft and publish a quarterly report satisfying the reporting obligations set in Articles 7, 20, 25 and 28 of this methodology:

- According to Article 7(3)(b), the CCC shall collect all reports analysing the effectiveness of relevant allocation constraints, received from the concerned TSOs during the period covered by the report, and annex those to the quarterly report – within scope of this report

- According to Article 20(13), the CCC shall provide all information on the reductions of cross-zonal capacity, with a supporting detailed analysis from the concerned TSOs where relevant – within scope of this report

- According to Article 25(2)(g), Core TSOs shall report on flows resulting from net positions resulting from SDAC on each CNEC and external constraint of the final flow-based parameters – within scope of this report

Additionally, the report covers reporting obligations from:

- Article 26(5) of Commission Regulation (EU) 2015/1222 of 24 July 2015 establishing a guideline on Capacity Allocation and Congestion Management (hereafter referred to as the “CACM regulation”)

- Article 16(3) of Regulation (EU) 2019/943 of the European Parliament and of the Council of 5 June 2019 on the internal market for electricity (hereafter referred to as the “IME regulation”)

The report follows the calendar quarters and consists of three main parts. Part 1 deals with allocation constraints, as per Article 7(3)(b); Part 2 deals with reductions of cross-zonal capacity, as per Article 20(13); Part 3 deals with flows resulting from net positions resulting from SDAC, as per Article 25(2)(g).

The report contains several raw data files (in the form of xlsx files), as well as Annexes. Annex I – QX_20XX_Allocation constraints addresses additional information provided by the Core TSOs that utilize allocation constraints, if applicable; Annex II – QX_20XX_Core TSOs measures for reductions addresses the Core TSOs proposed measures and action plans for avoiding future capacity reductions.

This document is meant as a reading guide, with the aim of introducing the reader to how all the reporting requirements are fulfilled and the location of each of the data item among the list of files attached.

# 1. Allocation constraints

## Reporting obligations from DA CCM

As per Article 7(3)(b) of the Core DA CCM:

“External constraints may be used by ELIA, TenneT B.V. and PSE during a transition period of two years following the implementation of this methodology in accordance with Article 28(3) and in accordance with the reasons and the methodology for the calculation of external constraints as specified in Annex 1 to this methodology. During this transition period, the concerned Core TSOs shall:

- In case the external constraint had a non-zero shadow price in more than 0.1% of hours in a quarter, provide to the CCC a report analysing: (i) for each DA CC MTU when the external constraint had a non-zero shadow price the loss in economic surplus due to external constraint and the effectiveness of the allocation constraint in preventing the violation of the underlying operational security limits and (ii) alternative solutions to address the underlying operational security limits. The CCC shall include this report as an annex in the quarterly report as defined in Article 27(5)”

The above reporting obligations are covered in this report as follows:

- Article 7(3)(b)(i): loss of economic surplus (for each DA CC MTU when the external constraint had a non-zero shadow price) – covered by the raw data file QX_20XX_Allocation constraints.xlsx and Annex I – QX_20XX_Allocation constraints.pdf

Note: There are 4 external constraints related to the DE-BE HVDC interconnector ALEGrO (BE_AL_import, BE_AL_export, DE_AL_import, DE_AL_export). These external constraints are of a different nature (they describe the 1000 MW technical capacity of the interconnector), thus they are not included in the allocation constraints-related data of this report.

## Reading guide for raw data

The raw data per MTU is available in the file QX_20XX_Allocation constraints.xlsx, worksheet raw data.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TimeInterval | YYYY-MM-DDTHH:mmZ | MTU, in UTC |
| TSO | text | TSO owner of the allocation or external constraint (ELIA, PSE and TenneT B.V.) |
| Shadow Price | Number (in €/MWh) | Shadow Price of external constraint for correspondent TimeInterval, in €/MWh |

Additionally, in the file QX_20XX_Allocation constraints_overview.xlsx, worksheet Overview, an overview is provided for each of the Core TSOs that use external or allocation constraints (ELIA, TenneT B.V. and PSE) on the percentage of DA CC MTUs when the external constraint had non-zero shadow price.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO | text | TSO owner of the allocation or external constraint (ELIA, PSE and TenneT B.V.) |
| Percentage of DA CC MTUs from analysed quarter when external constraint has non-zero shadow price [%] | Value (in %) | Value (in %) of DA CC MTUs from the analysed quarter when external constraint had a non-zero shadow price |

For any of the concerned TSOs, where there are more than 0.1% of MTUs when external constraint had non-zero shadow price, additional required information is provided in Annex I – QX_20XX_Allocation constraints, which contains information related to Article 7(3)(b)(i) as described in subchapter 1.1 Reporting obligations from DA CCM of this reading guide.

# 2. Reductions of cross-zonal capacity

## Reporting obligations from DA CCM

As per Article 20(13) of the Core DA CCM:

“Every three months, the CCC shall provide in the quarterly report all the information on the reductions of cross-zonal capacity, separately for coordinated and individual validations. The quarterly report shall include at least the following information for each CNEC of the pre-solved domain affected by a reduction and for each DA CC MTU:

- The identification of the CNEC

- All the corresponding flow components pursuant to Article 25(2)(d)(vii)

- The volume of reduction, the shadow price of the CNEC resulting from the SDAC and the estimated market loss of economic surplus due to the reduction

- The detailed reason(s) for reduction, including the operational security limit(s) that would have been violated without reductions, and under which circumstances they would have been violated

- If an internal network element with a specific contingency was exceptionally added to the final list of CNECs during validation: a justification why adding the network elements with a specific contingency to the list was the only way to ensure operational security, the name or the identifier of the internal network elements with a specific contingency, the DA CC MTUs for which the internal network elements with a specific contingency was added to the list and the information referred to in points (b) and (c) above

- The remedial actions included in the CGM before the day-ahead capacity calculation – reporting requirement fulfilled by reporting the initial PST tap settings before the Non-costly Remedial Action Optimisation phase, as per agreements with NRAs in Core IG calls 20200122 and 20200415

- In case of reduction due to individual validation, the TSO invoking the reduction

- The proposed measures to avoid similar reductions in the future”

Additionally, as per Article 20(14) of the Core DA CCM:

“The quarterly report shall also include at least the following aggregated information:

- Statistics on the number, causes, volume and estimated loss of economic surplus of applied reductions by different TSOs

- General measures to avoid cross-zonal capacity reductions in the future”

Finally, as per Article 20(15) of the Core DA CCM:

“When a given Core TSO reduces capacity for its CNECs in more than 1% of DA CC MTUs of the analysed quarter, the concerned TSO shall provide to the CCC a detailed report and action plan describing how such deviations are expected to be alleviated and solved in the future. This report and action plan shall be included as an annex to the quarterly report.”

Considering that for the analysed reporting period the Coordinated Validation process step is not in place, the scope of this report covers only capacity reductions applied in the Individual Validation process step.

In addition to the reporting obligations mentioned above, and in order for Core TSOs to improve the transparency and visibility of occurrences of RAM<20% Fmax.

The above reporting obligations are covered in this report as follows:

| Core DA CCM reporting obligation | Location of provided information |
| --- | --- |
| Article 20(13): |  |
| (a) | QX_20XX_Reductions.xlsx, worksheets: - Reductions_01 - Reductions_02 |
| (b) | QX_20XX_Reductions.xlsx, worksheet Reductions_02 |
| (c) | QX_20XX_Reductions.xlsx, worksheet Reductions_01 |
| (d) | QX_20XX_Reductions.xlsx, worksheet Reductions_01 |
| (f) | QX_20XX_Reductions.xlsx, worksheets: - Reductions_01 - Reductions_02 - Reductions_03 |
| (g) | QX_20XX_Reductions.xlsx, worksheet Reductions_04 |
| (h) | QX_20XX_Reductions.xlsx, worksheet Reductions_01 |
| (i) | QX_20XX_Reductions.xlsx, worksheet Reductions_01 |
| Article 20(14): |  |
| (a) | QX_20XX_Reductions-statistics.xlsx, worksheets: - Reductions-statistics_01 - Reductions-statistics_02 - Reductions-statistics_03 - Reductions-statistics_04_01 - Reductions-statistics_04_02 - Reductions-statistics_04_03 - Reductions-statistics_05_01 - Reductions-statistics_05_02 - Reductions-statistics_05_03 - Reductions-statistics_06_01 - Reductions-statistics_06_02 - Reductions-statistics_06_03 |
| (b) | Annex II – QX_20XX_Core TSOs measures for reductions.pdf |
| Article 20(15): |  |
| Indication of Core TSOs that applied reductions for more than 1% of DA CC MTUs | QX_20XX_Reductions-statistics.xlsx, worksheet Reductions-statistics_02 |
| Detailed report and action plan to avoid future reductions | Annex II – QX_20XX_Core TSOs measures for reductions.pdf |

The reporting obligations as per CACM Article 26(5) are covered in this report as follows:

| CACM Art. 26(5) reporting obligation | Corresponding Core DA CCM reporting obligation |
| --- | --- |
| Location of reductions in cross-zonal capacity | Article 20(13)(a) |
| Amount of reductions in cross-zonal capacity | Article 20(13)(c) |
| Reasons of reductions in cross-zonal capacity | Article 20(13)(d) |

The reporting obligations as per IME Article 16(3) are covered in this report as follows:

| IME Art. 16(3) reporting obligation | Corresponding Core DA CCM reporting obligation |
| --- | --- |
| Reductions of capacity | Reporting obligations related to reductions of cross-zonal capacity from individual validations – Article 20(13) |
| Deviations from coordinated actions | - |
| Assessment of incidences | Article 20(14)(a) |
| Recommendations | - |

The additional reporting information related to occurrences of RAM<20% Fmax is provided in this report as follows:

| Additional reporting information related to occurrences of RAM<20% | Location of provided information or corresponding Core DA CCM reporting obligation |
| --- | --- |
| Indication per CNEC and MTU of RAM value in percentage of Fmax | QX_20XX_Reductions.xlsx, worksheet Reductions_01 |
| Aggregated information per TSO: - CNECs with RAM<20% Fmax +3MW - Distinct MTUs with CNECs having RAM<20% Fmax + 3MW - CNECs with 0 RAM - Distinct MTUs with CNECs with 0 RAM | Article 20(14)(a) |

## Reading guide for raw data and statistics files

The raw data per MTU is available in the files mentioned below.

The raw data per MTU is available in the file QX_20XX_Reductions.xlsx. The worksheets are described below.

QX_20XX_Reductions_01

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| MTU (UTC) | YYYY-MM-DDTHH:mmZ | MTU, in UTC |
| TSO | text | TSO that applied respective reduction |
| CNEC name | text | Name of CNEC |
| CNEC ID | text | Unique identifier of CNEC |
| IVA [MW] | number | Volume of reduction (in MW) applied on respective CNEC from individual validation step |
| Share of IVA reducing the domain (Optional) | number | Share from volume of reduction (in MW) that effectively contracts the Flow-based domain; Applicable for DAVinCy TSOs (AT, German TSOs, NL) |
| Overloaded network element TSO (Optional if identical to CNEC with IVA) | text | TSO owning the forecasted overloaded network element Applicable for DAVinCy TSOs (AT, German TSOs, NL) |
| Overloaded network element name (Optional if identical to CNEC with IVA) | text | Name of the forecasted overloaded network element Applicable for DAVinCy TSOs (AT, German TSOs, NL) |
| Overloaded network element EIC Code (Optional if identical to CNEC with IVA) | text | EIC code of the forecasted overloaded network element Applicable for DAVinCy TSOs (AT, German TSOs, NL) |
| Overloaded network element UCT From Node (Optional if identical to CNEC with IVA) | text | UCT node From of the forecasted overloaded network element Applicable for DAVinCy TSOs (AT, German TSOs, NL) |
| Overloaded network element UCT To Node (Optional if identical to CNEC with IVA) | text | UCT node To of the forecasted overloaded network element Applicable for DAVinCy TSOs (AT, German TSOs, NL) |
| Contingency X TSO (Optional if identical to CNEC with IVA) | text | TSO owning the contingency X associated with the forecasted overloaded network element Applicable for DAVinCy TSOs (AT, German TSOs, NL) |
| Contingency X UCT From Node (Optional if identical to CNEC with IVA) | text | UCT node From of the contingency X associated with the forecasted overloaded network element Applicable for DAVinCy TSOs (AT, German TSOs, NL) |
| Contingency X UCT To Node (Optional if identical to CNEC with IVA) | text | UCT node To of the contingency X associated with the forecasted overloaded network element Applicable for DAVinCy TSOs (AT, German TSOs, NL) |
| Core NP X | text | Core Net Position of bidding zone X considered for the forecasted overloaded network element |
| Comment | text | Optional column with additional information regarding the respective applied reduction |
| Was fallback applied? | TRUE/FALSE | Indication of whether the respective applied reduction was applied as (1) as outcome of the individual validation tool (FALSE) or (2) as fallback, due to individual validation tool being unavailable (TRUE) |
| Measures to Avoid Future Reductions | text | Column with proposed measures to avoid future redutions for respective applied reduction |
| Shadow Price | Number (in €/MWh) | Shadow Price of CNEC affected by reduction, in €/MWh |
| Estimated market loss | Number (in €) | Estimated market loss of CNEC affected by reduction, in € The value for loss of economic surplus of applied reductions is approximated by multiplying the shadow price with the volume (in MW) of the applied reduction. This constitutes an upper estimate of the actual effect as price elasticity is ignored. |
| Presolved | TRUE/FALSE | If the value is TRUE then the corresponding Network Element with Contingency constraints the flow-based domain, FALSE means a redundant Network Element with Contingency not constraining the flow-based domain |
| RAM [% of Fmax] | Number, in % | Remaining Available Margin, expressed in percentages of Fmax, computed with the following formula: RAM0Core+AMR -CVA-IVA+3* FMax *Core TSOs have agreed to add a margin of 3 MW in the formula that represents the rounding that can occur during the process |

QX_20XX_Reductions_02

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TimeInterval | YYYY-MM-DDTHH:mmZ | MTU, in UTC |
| DeliveryDate | YYYY-MM-DD | Business Day |
| Period | Number (1..23/24/25) | Timestamp |
| NEC_ID | text | Unique identifier of Network Element with Contingency |
| NE_EIC | text | EIC code of the Network Element |
| NE_Name | text | Name of Network Element |
| NE_from | text | UCT node From of the Network Element |
| NE_to | text | UCT node To of the Network Element |
| NE_ElementName | text | Element Name of the Network Element |
| NE_OrderCode | number | Order Code of the Network Element |
| Contingency_ID | text | Unique identifier of Contingency |
| Contingency_Name | text | Name of Contingency |
| TSO | text | TSO owning the Network Element with Contingency |
| CNEC | TRUE/FALSE | Indication of type of Network Element with Contingency TRUE: Network Element with Contingency passed the 5% PTDF rule and will be part of the pre-solved dataset FALSE: Network Element with Contingency was filtered out by the 5% PTDF rule Only pre-solved CNECs (CNEC = TRUE) are reported, as per Art. 20(13)(b) of the Core DA CCM |
| MNEC | TRUE/FALSE | Indication of whether Network Element with Contingency is a monitored network element, as per the Core DA CCM |
| F_max | Number, in MW | Maximum allowable power flow on the corresponding Network Element with Contingency |
| FRM | Number, in MW | Flow Reliability Margin |
| F_ref | Number, in MW | Reference Flow |
| F_totalLTN | Number, in MW | Flow after consideration of LTN Ftotal_LTN = F0,core + F_LTN |
| RAM | Number, in MW | Remaining Available Margin |
| F_0core | Number, in MW | Flow per Network Element with Contingency in the situation without commercial exchanges within the Core CCR |
| RAM_0core | Number, in MW | Remaining Available Margin in the situation without commercial exchanges within the Core CCR |
| MinRAMFactor | Number, in % | Target for minimum margin for the totality of market exchanges (including non-Core exchanges) |
| F_uaf | Number, in MW | Flow resulting from assumed commercial exchanges outside the Core region |
| F_0all | Number, in MW | Flow per Network Element with Contingency in the situation without commercial exchanges between bidding zones within Continental Europe and between bidding zones within Continental Europe and bidding zones of other synchronous areas |
| F_LTN | Number, in MW | Flow resulting from Long Term Nominations |
| AMR | Number, in MW | Adjustment for minimum RAM |
| LTA_margin | Number, in MW | Flow margin for LTA inclusion LTA_margin = max(FLTAmax + FRM – AMR – Fmax; 0) |
| CVA | Number, in MW | Coordinated value adjustment resulting from coordinated validation process |
| IVA | Number, in MW | Individual value adjustment resulting from individual TSO validation process |
| Presolved | TRUE/FALSE | If the value is TRUE then the corresponding Network Element with Contingency constraints the flow-based domain, FALSE means a redundant Network Element with Contingency not constraining the flow-based domain Note: The file only contains entries of pre-solved CNECs (Presolved = TRUE), as required by the DA CCM |
| PTDF_XX | Number | One column per hub with the Power Transfer Distribution Factor value |

QX_20XX_Reductions_03

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TimeInterval | YYYY-MM-DDTHH:mmZ | MTU, in UTC |
| CNEC ID | text | Unique identifier of Network Element with Contingency exceptionally added |

QX_20XX_Reductions_04

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| MTU (UTC) | YYYY-MM-DDTHH:mmZ | MTU, in UTC |
| TSO | text | TSO applying initial PST tap settings |
| From node | text | UCT node From of the PST |
| To node | text | UCT node To of the PST |
| Element Name | text | Element Name of the PST |
| Initial PST Tap | Number | Initial PST tap setting |

Additionally, statistics/overviews on aggregated data are presented in the file QX_20XX_Reductions-statistics.xlsx. The worksheets are described below.

QX_20XX_ Reductions-statistics_01

In this worksheet, an overview on the following indicators:

- Number of BDs from the quarter

- Number of MTUs from the quarter

- Number of MTUs without applied reductions

- Number of MTUs with applied reductions

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| Total BDs from quarter | number | Number of BDs from the analysed quarter |
| Total MTUs from quarter | number | Number of MTUs from the analysed quarter |
| Total MTUs without applied reductions | number | Number of MTUs from the analysed quarter that have no reductions applied |
| Total MTUs with reductions applied | number | Number of MTUs from the analysed quarter that have reductions applied (from one or multiple TSOs) |

QX_20XX_ Reductions-statistics_02

In this worksheet, an overview is provided on the percentage of DA CC MTUs from the analysed quarter where each Core TSO has applied capacity reductions.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO | text | TSO that has applied capacity reductions in the analysed quarter |
| Percentage of DA CC MTUs from analysed quarter where Core TSO has applied capacity reductions [%] | Value (in %) | Value (in %) of DA CC MTUs from the analysed quarter where Core TSO has applied capacity reductions |

The following statistics are provided in below worksheets, as per Article Article 20(14)(a) of the Core DA CCM:

Number of applied reductions

QX_20XX_ Reductions-statistics_03

In this worksheet, an overview is provided on the number of applied reductions over the analysed reporting period. A distinction is made between total number of applied reductions and number of distinct MTUs with applied reductions, per TSO.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO | text | TSO that has applied capacity reductions in the analysed quarter |
| Number of applied reductions | Number | Number of applied capacity reductions from the analysed quarter, from respective TSO |
| Number of distinct MTUs with applied reductions | number | Number of distinct MTUs with applied capacity reductions from the analysed quarter, from respective TSO |
| Number of CNECs with RAM<20% Fmax + 3 MW | Number | Number of pre-solved CNECs with RAM<20% Fmax + 3 MW, from respective TSO |
| Number of distinct MTUs with CNECs with RAM<20% Fmax + 3 MW | Number | Number of distinct MTUs with pre-solved CNECs with RAM<20% Fmax + 3 MW, from respective TSO |
| Number of CNECs with 0 RAM | Number | Number of pre-solved CNECs with RAM=0, from respective TSO |
| Number of distinct MTUs with CNECs with 0 RAM | number | Number of distinct MTUs with pre-solved CNECs with RAM=0, from respective TSO |

Causes of applied reductions

QX_20XX_ Reductions-statistics_04_1

In this worksheet, an overview is provided on the number of applied reductions that occurred as a result of system security assessment performed by the TSOs individual validation tools vs the number of applied reductions due to individual validation tool being unavailable (and fallback options were used by TSOs to apply reductions).

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO | text | TSO that has applied capacity reductions in the analysed quarter |
| Reductions applied as outcome of individual validation tool | Number | Number of applied capacity reductions from the analysed quarter as outcome of the individual validation tool, from respective TSO |
| Reductions applied as fallback | Number | Number of applied capacity reductions from the analysed quarter as fallback (due to individual validation tool being unavailable) |

QX_20XX_ Reductions-statistics_04_2

Additionally for the first category from previous worksheet (reductions applied as a result of system assessment performed by the TSOs individual validation tools), in this worksheet an overview is provided on the number of applied reductions from a TSO due to own overloaded network element vs overloaded network element of another TSO; this is only relevant for the TSOs of Austria, Germany and the Netherlands, who perform the individual validation assessment jointly).

Since a single reduction can be related to overloaded elements of two or more TSOs (see footnote 3), reductions can be counted several times in this and all following statistics linking reductions to TSOs of the overloaded elements. This leads to a systematic overestimation of the figures for the DAVinCy TSOs.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO | text | TSO that has applied capacity reductions in the analysed quarter |
| Reductions applied for forecasted overloaded network element in TSO own area | Number | Number of applied capacity reductions from the analysed quarter when reduction was applied for forecasted overloaded network element in TSO own area, from respective TSO (as outcome of the individual validation tool) |
| Reductions applied for forecasted overloaded network element in other TSO area | Number | Number of applied capacity reductions from the analysed quarter when reduction was applied for forecasted overloaded network element in other TSO area, from respective TSO (as outcome of the individual validation tool) |

QX_20XX_ Reductions-statistics_04_3

Last worksheet on the causes of applied reductions focuses on the reductions applied by a TSO for a forecasted overloaded network element in a different TSO area (second category from previous file; this is only relevant for the TSOs of Austria, Germany and the Netherlands, who perform the individual validation assessment jointly). This worksheet provides a breakdown on number of applied reductions for a forecasted overloaded network element from own TSO area or from another TSO area.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO applying reductions | text | TSO A that has applied capacity reductions |
| TSO owning the forecasted overloaded network element | text | TSO B that owns the forecasted overloaded network element for which reduction was applied (can be the same as TSO A) |
| Occurrences | number | Number of applied reductions from TSO A due to forecasted overloaded network element from TSO B area |

Volume of applied reductions

Note: The “volume of applied reductions” refers to the IVA values. The TSOs of Austria, Germany and the Netherlands apply parts of their IVAs on CNECs that are not pre-solved. These CNECs are used as substitutes in cases when foreign CNECs, on which no IVA may be applied during individual validation, determine the market limits in the relevant directions(s). The IVA on the substitute CNECs is then partly needed to shift the CNEC to the edge of the CZC domain. Only the remainder of the IVA after this shift effectively reduces the size of the domain. Therefore, the “volume of reductions” (i.e. complete IVA) overstates the effective impact on the domain for these TSOs.

QX_20XX_ Reductions-statistics_05_1

In this worksheet, an overview is provided on the total volume of applied reductions that occurred as a result of system security assessment performed by the TSOs individual validation tools vs the total volume of applied reductions due to individual validation tool being unavailable (and fallback options were used by TSOs to apply reductions).

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO | text | TSO that has applied capacity reductions in the analysed quarter |
| Volume of reductions applied as outcome of individual validation tool | Number, in MW | Total volume (sum) of applied capacity reductions from the analysed quarter as outcome of the individual validation tool, from respective TSO |
| Volume of reductions applied as fallback | Number, in MW | Total volume (sum) of applied capacity reductions from the analysed quarter as fallback (due to individual validation tool being unavailable) |

QX_20XX_ Reductions-statistics_05_2

Additionally for the first category from previous worksheet (reductions applied as a result of system security assessment performed by the TSOs individual validation tools), in this worksheet an overview is provided on the total volume of applied reductions from a TSO due to own overloaded network element vs overloaded network element of another TSO.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO | text | TSO that has applied capacity reductions in the analysed quarter |
| Volume of reductions applied for forecasted overloaded network element in TSO own area | Number, in MW | Total volume (sum) of applied capacity reductions from the analysed quarter when reduction was applied for forecasted overloaded network element in TSO own area, from respective TSO (as outcome of the individual validation tool) |
| Volume of reductions applied for forecasted overloaded network element in other TSO area | Number, in MW | Total volume (sum) of applied capacity reductions from the analysed quarter when reduction was applied for forecasted overloaded network element in other TSO area, from respective TSO (as outcome of the individual validation tool) |

QX_20XX_ Reductions-statistics_05_3

Last worksheet on the volume of applied reductions focuses on the reductions applied by a TSO for a forecasted overloaded network element in a different TSO area (second category from previous file; this is only relevant for the TSOs of Austria, Germany and the Netherlands, who perform the individual validation assessment jointly). This worksheet provides a breakdown on the total volume of applied reductions for a forecasted overloaded network element from own TSO area or from another TSO area.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO applying reductions | text | TSO A that has applied capacity reductions |
| TSO owning the forecasted overloaded network element | text | TSO B that owns the forecasted overloaded network element for which reduction was applied (can be the same as TSO A) |
| Volume | Number, in MW | Total volume (sum) of applied reductions from TSO A due to forecasted overloaded network element from TSO B area |

Estimated loss of economic surplus of applied reductions

In the report, the value for loss of economic surplus of applied reductions on a CNEC in an MTU is approximated by multiplying the shadow price with the volume (in MW) of the applied reduction. It is to be noted that this approximation provides an upper bound of the actual loss of economic surplus – it assumes inelastic supply/demand prices of infinite amount (i.e. absence of partial or even full price convergence).

Moreover, the following aggregated figures are determined by adding up the CNEC-wise estimates from the raw data. However, a smaller capacity reduction on a given CNEC would reduce the shadow prices of all other CNECs (with non-zero shadow price) in the same MTU. This means that lowering reductions on two CNECs in an MTU would yield less market gain than the sum of the individual CNECs’ estimates (which are already upper estimates, see previous paragraph).

In addition, the remark concerning the reductions applied by the TSOs of Austria, Germany and the Netherlands, as mentioned in the previous section “Volume of applied reductions”, leads to overstating of the impact on these reductions on the market.

Due to the aforementioned effects the statistics systematically overestimate the actual total market loss of the applied reductions to an unknown extent.

QX_20XX_ Reductions-statistics_06_1

In this worksheet, an overview is provided on the total estimated loss of economic surplus of applied reductions that occurred as a result of system security assessment performed by the TSOs individual validation tools vs the estimated loss of economic surplus of applied reductions due to individual validation tool being unavailable (and fallback options were used by TSOs to apply reductions).

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO | text | TSO that has applied capacity reductions in the analysed quarter |
| Estimated market loss of reductions applied as outcome of individual validation tool | Number, in k€ | Estimated market loss of applied capacity reductions from the analysed quarter as outcome of the individual validation tool, from respective TSO |
| Estimated market loss of reductions applied as fallback | Number, in k€ | Estimated market loss of applied capacity reductions from the analysed quarter as fallback (due to individual validation tool being unavailable) |

QX_20XX_ Reductions-statistics_06_2

Additionally for the first category from previous worksheet (reductions applied as a result of system security assessment performed by the TSOs individual validation tools), in this worksheet an overview is provided on the estimated market loss of applied reductions from a TSO due to own overloaded network element vs overloaded network element of another TSO.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO | text | TSO that has applied capacity reductions in the analysed quarter |
| Estimated market loss of reductions applied for forecasted overloaded network element in TSO own area | Number, in k€ | Estimated market loss of applied capacity reductions from the analysed quarter when reduction was applied for forecasted overloaded network element in TSO own area, from respective TSO (as outcome of the individual validation tool) |
| Estimated market loss of reductions applied for forecasted overloaded network element in other TSO area | Number, in k€ | Estimated market loss of applied capacity reductions from the analysed quarter when reduction was applied for forecasted overloaded network element in other TSO area, from respective TSO (as outcome of the individual validation tool) |

QX_20XX_ Reductions-statistics_06_3

Last worksheet on the estimated market loss of applied reductions focuses on the reductions applied by a TSO for a forecasted overloaded network element in a different TSO area (second category from previous file; this is only relevant for the TSOs of Austria, Germany and the Netherlands, who perform the individual validation assessment jointly). This worksheet provides a breakdown on the estimated market loss of applied reductions for a forecasted overloaded network element from own TSO area or from another TSO area.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TSO applying reductions | text | TSO A that has applied capacity reductions |
| TSO owning the forecasted overloaded network element | text | TSO B that owns the forecasted overloaded network element for which reduction was applied (can be the same as TSO A) |
| Estimated market loss | Number, in k€ | Estimated market loss of applied reductions from TSO A due to forecasted overloaded network element from TSO B area |

# 3. Flows resulting from net positions resulting from SDAC

## Reporting obligations from DA CCM

As per Article 25(2)(g) of the Core DA CCM:

“The CCC shall include in its quarterly report as defined in Article 27(5) the flows resulting from net positions resulting from the SDAC on each CNEC and external constraint of the final flow-based parameters.”

The above reporting obligation is covered in this report by the raw data file QX_20XX_Resulting-flows-SDAC.csv

## Reading guide for raw data

The raw data per MTU is available in the file QX_20XX_Resulting-flows-SDAC.xlsx.

The following columns are included, with the respective column description, expected format.

| Column name | Expected format | Description |
| --- | --- | --- |
| TimeInterval | YYYY-MM-DDTHH:mmZ | MTU, in UTC |
| DeliveryDate | YYYYMMDD | Business Day |
| Period | Number (1..23/24/25) | Timestamp |
| NEC_ID | text | Unique identifier of Network Element with Contingency |
| NE_EIC | text | EIC code of the Network Element |
| NE_Name | text | Name of Network Element |
| NE_from | text | UCT node From of the Network Element |
| NE_to | text | UCT node To of the Network Element |
| NE_ElementName | text | Element Name of the Network Element |
| NE_OrderCode | number | Order Code of the Network Element |
| Contingency_ID | text | Unique identifier of Contingency |
| Contingency_Name | text | Name of Contingency |
| TSO | text | TSO owning the Network Element with Contingency |
| CNEC | TRUE/FALSE | Indication of type of Network Element with Contingency TRUE: Network Element with Contingency passed the 5% PTDF rule and will be part of the pre-solved dataset FALSE: Network Element with Contingency was filtered out by the 5% PTDF rule |
| Resulting_Flow_SDAC | Number, in MW | Flow after consideration of SDAC Net Positions |
| F_totalLTN | Number, in MW | Flow after consideration of LTN Ftotal_LTN = F0,core + F_LTN |
| Ftotal(SDAC+LTN) | Number, in MW | Flow resulting from Net Positions resulting from SDAC Computed with following formula: Resulting_Flow_SDAC + F_totalLTN |
| Presolved | TRUE/FALSE | If the value is TRUE then the corresponding Network Element with Contingency constraints the flow-based domain, FALSE means a redundant Network Element with Contingency not constraining the flow-based domain |
| PTDF_XX | Number | One column per hub with the Power Transfer Distribution Factor value |
| NP_XX | Number | One column per hub with the SDAC Net Position value |

Obligation from Art. 27(5)(c) is applicable only from beginning of parallel run until Core FB MC go-live, thus not in scope of this report

Following reporting requirements were moved to the annual report, following agreement with Core NRAs in Core IG call 20200415:

- Article 7(3)(b)(i): effectiveness of the allocation constraint in preventing the violation of the underlying operational security limits

- Article 7(3)(b)(ii): alternative solutions to address the underlying operational security limits

In the Core DA CCM 1st amendment, Art. 20(13)(e) was removed.

When the DAVinCy validation process identifies the need to contract the flow-based domain in a direction where it is bounded by a foreign CNEC, IVA is applied on a “substitute” CNEC from a DAVinCy TSO instead. As the substitute CNEC is initially outside of the domain, a part of its IVA is needed to shift it to the edge of the domain, and only the remainder is effectively contracting the domain.

The DAVinCy individual validation process is jointly performed by the Austrian, German and Dutch TSOs. Due to simulated application of remedial actions, network element/contingency combinations (be it CNECs or not) whose overloading cannot be prevented by all available remedial actions can differ from the CNECs of the flow-based domain that are needed to restrict the cross-zonal capacity, such that such overloading is prevented. This is because the flow-based domain is based on the zonal GLSK model, whereas DAVinCy performs generator-precise redispatch to optimally level the network loading in order to restrict the reduction of RAM to the minimum extent necessary. As a consequence, overloading of a network element/contingency combination in the grid of one TSO may be addressed by IVA on a CNEC of another TSO. Moreover, network elements that are no CNECs might give rise to the need for a capacity reduction in case their overloading cannot be prevented by all available remedial actions. For these reasons, the overloaded network elements and contingencies are reported separately from the CNECs on which the reductions were applied. Consequently, when reporting reductions and their relation to the TSOs owning the overloaded elements, a single reduction can be counted several times (namely if overloaded elements causing that reductions were located in the areas of several TSOs).