# Annex II – Core TSOs general measures and action plan to avoid future cross-zonal capacity reductions

Q1 2026

Table of Contents

Introduction	2

1.	CEPS	3

2.	DAVinCy TSOs	4

2.1.	APG	4

2.2.	German TSOs	4

2.3.	TENNET TSO BV	4

3.	ELES	4

4.	ELIA	4

5.	HOPS	5

6.	MAVIR	6

7.	PSE	6

8.	RTE	6

9.	SEPS	6

10.	Transelectrica	6

# Introduction

This annex contains the required information for each Core TSO that has applied capacity reductions for its CNECs in more than 1% of ID CC MTUs of the analysed quarter as described per Article 18(12).

# CEPS

In Q1 2026, ČEPS applied the IVA reduction within the IDCC(c) process in only five cases. This represents a significantly lower number compared to previous quarters and highlights a noticeable decline in the frequency of such adjustments. Although the first quarter of the year is generally characterized by higher system loading and increased operational stress due to seasonal consumption patterns, it is also typically associated with a lower number of planned maintenance activities and transmission line outages. The reduced extent of grid maintenance and network unavailability during this period likely contributed to more stable system conditions and decreased necessity for applying IVA reductions within the IDCC(c) framework. Overall, the Q1 2026 figures indicate a substantial reduction in the utilization of this mechanism compared to historical trends observed in earlier quarters.

# DAVinCy TSOs

## APG

Overview

ATC reductions are essential to prevent network overloads due to the challenges outlined below along with their mitigation measures.

For IDCC(c), APG performs a dual-validation approach is applied, combining ATC validation with IVA validation in coordination with six iDaVincy TSOs.

APG’s geographic location and outdated trading results from non-Core bidding zones

APG’s central position in the interconnected European grid, belonging to three capacity calculation regions (Core und Italy North), and its proximity to Switzerland and the Balkans, increases APG’s CNEC loading sensitivity to deviations between forecasted and actual conditions as well as misalignments between the regions. This unique position amplifies the impact of outdated market data.

In general, enhanced synchronization across CCRs can ensure more consistent and reliable capacity calculations, which could prevent mismatches and reduce the frequent triggering of local validations. The introduction of the CE CCR, that will cover the Core and the Italy North CCRs, represents a significant step toward more accurate capacities.

Neglect CNECs with small PTDF and RAM combinations.

The central ATC extraction algorithm excludes CNECs with RAM below 50 MW for bidding zone border where their PTDF is ≤ 3%. This can result in the allocation of non-existent capacities. To mitigate this and ensure operational grid security, APG validates all Austrian CNECs.

Need for limitations equal or higher than initial ATCs due to ATC redistribution

During the ATC validation process, APG can only assess whether the given initial ATCs result in a combination of ATCs that do not overload any APG CNEC. However, due to the central calculation design, a redistribution of ATCs after validation is possible. This redistribution can lead to a completely new set of ATCs that were not known during the validation phase.

As a result, even when the initial ATCs pose no risk, APG determinates validated ATCs that are equal or higher than the initial ATCs. These validated ATCs can subsequently limit the final ATCs in case of significant increases caused by redistribution.

This approach is particularly important given the impact of neglected flows, as it ensures grid security despite potential redistributions.

Unlike ATC validation, IVA validation can only result in lower FB capacities. Thus, a validated initial domain that does not compromise grid security will remain secure after the validation step because the IVA process effectively restricts the FB domain to ensure safety.

Conclusion

APG is actively addressing the challenges associated with IDCC calculations. By improving market data synchronization, enhancing validation processes assumptions, and contributing to broader capacity calculation advancements, APG is working to minimize the need for capacity reductions while ensuring operational security and market efficiency. Furthermore, as of early Q2 2026 it is possible to consider the latest market changes in the non-Core SDAC/SIDC bidding zones.

## German TSOs

## TENNET TSO BV

# ELES

General measures to avoid cross-zonal capacity reductions in the future, as per Article 18(11)(b) of ID CCM:

Improvement in congestion management – we will continue to improve the quality of our inputs for CORE ID CC in order to avoid unnecessary IVA application in case of errors in inputs files.

Network development and optimisation.

Detailed report and action plan describing how such deviations are expected to be alleviated and solved in the future as per Article 18(10)(h)(i) of ID CCM

In Q1 of 2026, ELES applied reduction in 19,70 % of MTU. 65 % of these reductions were applied on CNECs on AT-SI and HU-SI borders (Kainachtall – Maribor 1&2, Maribor-Cirkovce 1&2 and Cirkovce-Heviz). The remaining 35% of applied reductions were applied on CNECs on the HR-SI border (Divaca-Pehlin and Krsko-Tumbri 1&2). The main reason for the above reductions at ELES lies in the fact, that we are in the cross-road of two CCR, which both aim to maximise capacities in order to fulfil 70% criteria in DA. Often, maximisation of the capacities in Italy North CCR have negative effect on the RAM of Slovenian elements in CORE CCR. This is mostly due to the fact, that we have a PST on the Slovenian – Italian border, that is used to maximise Italy North NTC values. Maximising capacities in DA often translates to low capacities available for Intraday.

Our plan to improve the situation consists of the following:

Additional training of operators and improvement of the local validation tool in order to improve the process and improve stability and reliability of the tool

Analysis will be performed on accuracy of validation tool (e.g. comparing the flows considered during the validation and realised flows). Based on the result of the analysis, the validation tool reliability margin will be adjusted in order to decrease the level of IVA application.

Network development and optimisation - For AT-SI border, we are in the process to obtain and install a static serous synchronous compensator (SSSC) in Podlog substation (currently planned for Q4 2026 or early 2027) in order to be able to redistribute and relieve the flows on the elements Kainachtal – Maribor and Obersielach- Podlog. For the Divaca – Pehlin line, we are still investigation different possibilities to increase capacities (SSSC or high temperature lines).

Improve the outage planning coordination.

# ELIA

# HOPS

General measures to avoid cross-zonal capacity reductions in the future, as per Articles 18(10)(h)(i) and 20(11)(b) of the ID CCM

General measures include, but are not limited to:

· Network development and optimization

The goal is to increase the transmission capacity and reduce grid congestion. The measures to achieve these goals include strengthening and optimizing the existing network and the development of new infrastructure.

· Improvements concerning congestion management

Core CCR coordinated improvements with coordinated actions to increase cross-zonal capacities (for example improvements of the outage planning coordination in order to increase flexibility of the grid). Introduction of additional effective remedial actions should help to relieve the congestion and therefore allows to reduce the number of IVA application. Also, inclusion of third countries could open further opportunities for HOPS (with planning process and implementation of remedial measures). Unscheduled allocated flows coming from commercial exchanges outside the Core CCR (Fuaf) has a strong impact on HOPS grid.

Detailed report and action plan describing how such deviations are expected to be alleviated and solved in the future

In the analysed quarter (Q1 2026), HOPS applied reduction to around 8,52% MTUs. For most MTUs, the reductions are applied to:

· TL 220kV Zakucac - Mostar (0,06 % MTUs, or around 0,72% of times of all HOPS applied reductions)

· TL  220kV Pehlin - Divaca (8,09% MTUs, or around 94,93% of times of all HOPS applied reductions)

· TL  400kV Melina - Divaca (0,49% MTUs, or around 5,80% of times of all HOPS applied reductions)

· TL  400kV Tumbri - Krsko 1 (0,12% MTUs, or around 1,45% of times of all HOPS applied reductions)

· TL  400kV Tumbri - Krsko 2 (0,12% MTUs, or around 1,45% of times of all HOPS applied reductions)

Applied reductions on network element are mostly low (less than 2,04% of Fmax), while for a few MTUs during January, February and March slightly higher values are applied on TL 220kV Pehlin - Divaca (BD20260315 & BD20260120) and TL 400kV Tumbri - Krsko 1 and 2 (BD20260204) due to unsolvable overloads in the relevant grid area caused by unavailability of grid elements in the surrounding area, additionally under the influence of high exchanges between Core and non-Core countries (impact of uncoordinated flows coming from third countries, mainly from the direction of RS and BA). It is important to emphasize that energy needs increased during Q1 2026 in SEE with significant flows in the network towards SEE, which is why some reductions were applied during Q1 2026 to maintain a secured network. Such reductions are planned to be solved by developing and optimising the transmission network.

In Q2 of 2025, HOPS installed HTLS conductors on OHL 220 kV Brinje – VE Pađene that increases its maximum admissible power flow and improves available capacities, , while the upgrade continued in Q4 2025 and Q1 2026 on OHL 220 kV Brinje – VE Pađene and OHL 220 kV Konjsko – Pađene after which the entire Brinje – VE Pađene – Konjsko section will have HTLS installed.

Also, following the ongoing investigation of various options for increasing capacity for the TL 220 kV Pehlin – Divača, it is planned to start installing HTLS conductors on TL 220 kV Pehlin – Divača in Q4 2026.

Improvements are also expected with upcoming important processes such as coordinated validation capacities and Regional Operational Security Coordination.

# MAVIR

# PSE

# RTE

# SEPS

# Transelectrica

In addition to being part of the Core CCR, Transelectrica is also part of SEE CCR with RO – BG border having operational processes for the First Intraday capacity calculation since October 2021 and Second Intraday capacity calculation since October 2022. Moreover, there are three non-EU borders for which there is no coordinated capacity calculation.

Regarding the capacity calculation process, the Core ID CCM allows TSOs to correct cross-zonal capacity for reasons of operational security during the validation process individually and in a coordinated way according to Article 18 (2): “Each Core TSO shall validate and have the right to decrease the RAM for reasons of operational security during individual validation. [...] IVA may reduce the RAM only to the minimum degree that is needed to ensure operational security, and only after all the expected available costly and non-costly remedial actions pursuant to Article 22 of the SO Regulation are considered”.

As of 2026, the Action Plan adopted pursuant to Article 15 of Regulation (EU) 2019/943 ceased to apply. For the first half of 2026, Transelectrica submitted a derogation request based on a fixed minimum capacity level of 48% of the transmission capacity applicable to all elements, with the exception of the following two critical network elements, for which a fixed minimum capacity level of 41% of the transmission capacity was requested:

220 kV OHL Reșița – Timișoara d.c., in the direction from 220 kV Reșița substation to 220 kV Timișoara substation;

220 kV OHL Târgu Jiu Nord – Paroșeni, in the direction from 220 kV Târgu Jiu Nord substation to 220 kV Paroșeni substation.

This represents an increase from 41% to 48% for the majority of elements, in the absence of additional network investments. This increase reflects Transelectrica’s commitment to move closer to the 70% target by allowing higher cross-zonal capacities whenever compatible with operational security, while accepting a more constrained operational margin. Starting from the third quarter of 2026, a dynamic minimum capacity value is planned to be implemented.

For the period 20260101 – 20260331, Transelectrica applied an Individual Validation Adjustment (IVA) on the following CNEs, for a total of 8.7% of MTUs. In the graph below the average IVA per CNE is represented for the timestamps where reductions were applied.

OHL 400 kV Porțile de Fier – Djerdap is a tieline between Romania and Serbia, both countries having tielines with Bulgaria and Hungary. Because of this, the power flows on OHL 400 kV Porțile de Fier – Djerdap are highly impacted by the cross-border exchanges between RO-BG, RO-RS, RO-HU.

General measures to avoid cross-zonal capacity reductions in the future, as per Articles 18(10)(h)(i) and 20(11)(b) of the ID CCM are described below.

Development of the transmission grid

The main measure to reduce overloads in the capacity calculation process is to finalize investment projects with cross-border impact:

New OHL 400 kV Porțile de Fier – Reșița and TIE 400 kV Reșița – Pancevo circ. 2 have been commissioned in mid-November 2024. TIE 400 kV Reșița – Pancevo circ. 1 was commissioned in February 2025.

400 kV OHL Reșița – Timișoara – Săcălaz to be commissioned in 2027;

New Autotransformer 400/220 kV in Roșiori substation to be commissioned in 2028;

400 kV d.c. OHL Timișoara – Arad to be commissioned in 2028;

Increasing the transmission capacity on the OHLs 220 kV Urechești – Târgu Jiu Nord – Paroșeni – Baru Mare – Hășdat to be commissioned in 2028;

Increasing the transmission capacity on the OHL 220 kV d.c. Porțile de Fier – Reșița to be commissioned in 2028;

400 kV TIE Nădab – Grăniceri circ. 2 to be commissioned in 2028;

400 kV TIE Porțile de Fier – Djerdap circ.2 to be commissioned in 2029;

400 kV TIE Oradea Sud – Josza to be commissioned in 2030.

Outage planning coordination

Relevant assets are coordinated in the Outage coordination regions. This coordination has the scope of avoiding simultaneous disconnection of multiple elements with cross-border impact, taking into consideration the system security as a first step. Lately it has been observed that the finalization of the Yearly Maintenance Plan is highly impacted by the level of cross-border capacities provided to the market. Thus, it became a necessity to coordinate the outage of relevant assets also from the perspective of the capacity calculation processes.

Implementation of regional coordinated processes for security analysis and redispatching and countertrading processes

Results of the intraday capacity calculation process come with a lot of uncertainties, mainly due to the input data, volumes of capacity allocated and available redispatching. Measures to decrease the need of application of IVA include the consideration of internal redispatching. Due to the proximity to real-time, multiple power plants cannot be considered. Also, depending on the location of the congested elements, there may be none, one or multiple generators that can help decreasing the overload, but not necessarily avoiding them altogether.

It is thus mandatory to have security analysis implemented at regional level before real-time with coordinated means of reducing the observed overloads.

At Core CCR level these processes are still under implementation.